Request for quote details.

Carport type structure comprising semi open bay and one closed storage area – basic indicative sketch below.

Structure to be sited on pre-existing carport concrete slab – 5600mm x 3500mm.

Site located at Cowes (Phillip Island, Vic) and 2kms from marine environment.

Wall ‘A’ – Front elevation: 2600mm eave height

Wall ‘B’ – Rear elevation: 2300mm eave height

End Wall ‘C’ – Left elevation.

End Wall ‘D’ – Right elevation.

Skillion roof – 5 degrees pitch with wall B rear elevation height - 2300mm.

SHS Duragal columns (one piece and not sleeved) with base plates for securing to slab.

All roofing/wall cladding and flashing to be in Colorbond colour: ‘Monument‘ top side and presumably the default underside colour is ‘Thredbo White’ or similar light colour.

Roof cladding: High rib profile comparable to ‘Stramit Monoclad’

External walls and internal partition: Low rib profile version of ‘Stramit Monoclad’.

Wall cladding to be in vertical orientation and to sit flush around perimeter of slab and minimum 25mm below existing slab height.

Roof eave along Wall A to extend/overhang by 500mm.

Structure to comprise two bays:

Bay 1:

- 3500mm x 2400mm.

- Will be fully enclosed.

- To include 1 x Standard PA door in ‘Low Profile’ cladding incorporated into partition wall. Door outward opening (left-to-right) with hinging as near as practical to wall A.

- Full length polycarbonate skylight sheeting centrally positioned in bay 1 in profile corresponding to roofing cladding.

Bay 2:

- 3500mm x 3200mm.

- Open on two sides (ie wall A and end wall C).

Corner/fascia/barge flashing

Quad guttering – bracket fitting.

Demolition of existing carport and new construction by owner.

Options/other considerations

  - Aluminium framed sliding window (approx. 800mm x 900mm or near standard size) centrally positioned in wall A of bay 1 and approx. 1200mm from slab.

- Provision of full length polycarbonate skylight sheeting in bay 2.

- To extend Wall A from bay 1 into Bay 2 area by 900mm.

- Subject to sizing options available, would consider wider PA door than standard 820mm.

Questions/Other Information.

- I am intending to initially utilise the structure as a carport (phase 1). Consequently, would not be erecting end wall D, nor the internal partition wall comprising the personal access door until a later date when Bay 1 would be created as a fully enclosed storage area (phase 2).   Therefore, can end wall D and internal partitioning/ PA door be retro-fitted without compromising structural integrity at the phase 1 stage.  If deemed particularly beneficial, the upper wall girts of both the partition and end walls D could be installed as part of phase 1 to limit any negative lateral structural impacts.

The overall intent is to purchase the total structure as a package.